| MythSim Microcode Language Quick Reference |  |  |  |
| --- | --- | --- | --- |
| Register Set |  |  |  |
| a_sel=n |  | Place the value in register n (0-7) on the a_bus. |  |
| b_sel=n |  | Place the value in register n (0-7) on the b_bus. |  |
| ri_sel |  | Store result in the register defined in ir_ri. |  |
| rj_sel |  | Place the value in the register defined by ir_rj on the a_bus. |  |
| rk_sel |  | Place the value in the register defined by ir_rk on the b_bus. |  |
| rn_write |  | Store the result_bus value into register n (0-7). |  |
| Arithmetic Logic Unit |  |  |  |
| alu_sel= | NOT | NOT(a_bus) => alu_bus |  |
|  | OR | OR(a_bus, b_bus) => alu_bus |  |
|  | AND | AND(a_bus, b_bus) => alu_bus |  |
|  | XOR | XOR(a_bus, b_bus) => alu_bus |  |
|  | ADD | a_bus + b_bus + c_in => alu_bus |  |
|  | SUB | a_bus + NOT(b_bus) + c_in => alu_bus |  |
|  | ADDA | a_bus + c_in => alu_bus |  |
|  | SUBA | a_bus -1 + c_in => alu_bus |  |
| c_in |  | Set the c_in to 1 for use by ALU operations (4-7). |  |
| Memory Interface |  |  |  |
| result_sel= | ALU | Place the alu_bus value on the result_bus. |  |
|  | MDR | Place the mdr value on the result_bus. |  |
|  | IR_CONST4 | Place ir_const4 on the result_bus. (ir_const4 = last 4 bits of ir0) |  |
|  | IR_CONST8 | Place ir_const8 on the result_bus. (ir_const8 = all 8 bits of ir0) |  |
| ir0_sel= | LOAD | Load the value on the memory_bus into ir0 (Instruction Register 0). |  |
| ir1_sel= | LOAD | Load the value on the memory_bus into ir1 (Instruction Register 1). |  |
| mar_sel= | LOAD | Load the value on the alu_bus into the mar (Memory Address Register). |  |
| mdr_sel= | LOAD_ALU | Load the value on the alu_bus into the mdr (Memory Address Register). |  |
|  | LOAD_MEM | Load the value on the memory_bus into the mdr (Memory Data Register). |  |
| read |  | Place the value at the main memory address mar on the memory_bus. |  |
| write |  | Write the value of mdr to the main memory location mar. |  |
| Control Structures |  |  |  |
| goto label |  | Jump to the given label. |  |
| if status_line then goto label_1 (else goto label_2) endif |  | If the value of status_line=1 then goto label_1 else goto label_2. |  |
|  |  | status_line | Description |
|  |  | m7 | most significant bit from alu_bus |
|  |  | c_out | carry bit from the alu_bus |
|  |  | v | overflow bit from the ALU |
|  |  | wait | wait signal from Main Memory |

Control Unit Diagram

Datapath Diagram